# The interplay between clay fabric and mechanical response of deep-seated landslides

C. Seguí1, E. Tauler2, X. Planas3, J. Moya4, and M. Veveakis1

1Duke University, Civil and Environmental Engineering Department, Durham, USA.

2Universitat de Barcelona, Departament de Mineralogia, Petrologia i Geologia Aplicada, Barcelona, Spain.

3Ministeri d’Economia i Territori, Departament d’Ordenament Territorial, Andorra la Vella, Andorra.

4Universitat Politècnica de Catalunya, Departament d’Enginyeria Civil i Ambiental, Barcelona, Spain.

Corresponding author: Carolina Seguí (carolina.segui@duke.edu)

# Abstract

Deep-seated landslides are amongst the most devastating natural hazards on earth, typically involving a rigid rock mass sliding over a weak, clayey shear-band. The mechanical response of this shear-band to the loading of the overburden is therefore critical for the stability of a landslide. We hereby show that this mechanical response is strongly linked to the mineralogy and microstructure of the clay minerals forming the shear band. By presenting a detailed mineralogical, textural, and mechanical characterization of a shear band of an active deep-seated landslide we attempt to shed light on the processes determining the failure mechanism of a large deep-seated landslide.

The case study chosen is the El Forn landslide located above the village of Canillo (Andorra). Its shear band is at 29m depth and is formed by black shales of the Silurian period. Using core samples of this landslide, we have performed compositional and microstructural (XRPD, SEM-EDS, MicroCT) analyses, combined with mechanical tests (Liquid Limit, Plastic Limit) to study the interplay between the internal structure of the material that forms the shear band and its mechanical response. The results show that the mechanical properties (porosity and plasticity index) have their highest values inside the shear band, where the clay fabric is perfectly oriented along the shearing direction. Right above and below the shear band, the mechanical parameters have their lowest values, and are accompanied by folded textures (fabric). Finally, outside those areas, the mechanical parameters have intermediate values and the fabric of the minerals is randomly oriented.

# Introduction

The shear band of a large deep-seated landslide is the weakest part of the landslide [Kilburn and Petley, 2003], admitting chemical and mechanical processes that alter the material and trigger the movement of the sliding mass from its creeping phase to its collapse [Veveakis et al., 2007]. Large deep-seated landslides can involve millions of cubic meters of rock mass, threating infrastructure and human life in its path, thus creating the need for a deeper knowledge of the processes that happen inside the shear band. Deep-seated landslides can fail because of: 1) external factors, changing the loading stress of the slope beyond its static limit. Such factors include the increase of groundwater level due to rainfall or artificial dam impoundment, and earthquakes; and 2) internal factors that change the shearing resistance of the shear-band material (i.e., strength and friction coefficient) without any variation of the loading stress. Such factors include degradation processes including thermal, hydraulic and chemical effects [Vardoulakis, 2000, 2002a; Chang et al., 2005a; Veveakis et al., 2007; Goren and Aharonov, 2007; Regenauer-Lieb et al., 2009; Goren et al., 2010; Cecinato et al., 2011].

Some of the most prominent effects that could reduce the friction coefficient of the shear band material, for example, are thermal in origin [Anderson, 1980; Voight and Faust, 1982; Lachenbruch, 1980; Mase & Smith, 1984; Vardoulakis, 2002; Rice, 2006]. These mechanisms are of primary importance to thermally sensitive materials like clays, making thus very important the knowledge of the material forming the shear band of the landslide and this material’s behavior when affected by large deformation and stresses [Vardoulakis, 2002]. It is very common in large deep-seated landslides to have a shear band formed by clays or clay-like material. This kind of material is challenging with respect to its response under shear, due to its mineralogical composition (i.e., phyllosilicates), especially when these minerals are in contact with water (i.e., groundwater). Saturated clays can become very plastic, which leads to sudden collapses of large deep-seated landslides after years of creeping, as the case of the Vaiont landslide (Italy, 1963) [Veveakis et al., 2007; Segui et al., 2019].

Despite the importance of the microstructure of clay minerals to their macroscopic response, only limited micro-structural studies on clay/clay-like materials forming landslide shear-bands have been performed. The majority of them, have been studied for faults located in plate margins [Labaume et al., 1997; Chester and Logan, 1987; Colletini et al., 2009], and one for the Walton’s Wood landslide [Tchalenko, 1968]. All of these studies show that inside the shear band the phyllosilicates are fully oriented and aligned parallel to the shear movement, and outside the shear band the fabric of the clays is randomly oriented. All of them also agreed that the alignment of the clay minerals inside the shear band affect the shearing resistance of the material, decreasing its friction coefficient [Labaume et al., 1997; Chester and Logan, 1987; Colletini et al., 2009; Tchalenko, 1968]. Maltman [1987] studied experimentally the microstructure of a clayey shear band and stated that the alignment of the phyllosilicates in the shear band is not due to the flow of the groundwater but due to an intense shear movement. Labaume et al. [1997] linked the orientation of the phyllosilicates inside the shear band with a compactant plastic strain behavior of the clays.

Studies performed regarding the possible failure mechanisms of large deep-seated landslides focusing on the shear band behavior include mechanical parameters of the clayey material of the shear band and its response in to external factors (such as groundwater variations) [Vardoulakis, 2000, 2002a; Veveakis et al., 2007; Goren and Aharonov, 2007; DeBlasio and Elverhoi, 2008; Goren et al., 2010; Regenauer-Lieb et al., 2009; Chang et al., 2005a; Ferri et al., 2011; Cecinato and Zervos, 2012; Segui et al., 2019]. Moreover, the works that have focused on the current case study - El Forn landslide (Andorra) – have not included response of the shear band material and its impact on landslide failure. Firstly, Corominas and Alonso [1984] and Soutadé [1988], presented the landslide and its history, respectively. Corominas [1990] described its slope stability, Santacana [1994] and Planas et al. [2011] presented the history of movements occurred in the landslide. More recently, Jaqués [2014] proposed the latest geological model of the landslide with a new history of movements of the slide and an analysis of slope stability. Jaqués [2015] also studied the general and local behavior of the active landslide. Finally, Corominas et al., [2015] followed the sliding movement by radar and differential interferometry techniques.

The present work extends these studies by focusing on the role of the microstructural response (fabric) of clays in the macroscopic response of deep-seated landslides. We presenting as a case study the El Forn landslide (Andorra), which is the largest active deep-seated landslide in Andorra country, involving a mass of approximately 300 Mm3 [Corominas et al., 2013]. In the following sections, we firstly present the case study of El Forn landslide, before detailing the methods used for the microstructural and mechanical characterization of the landslide’s materials. Following this, the results of these studies, in combination with appropriate field data are presented, and thoroughly discussed.

# Case study: The El Forn Landslide

The El Forn landslide is the largest landslide in Andorra country (Figure 1A), on the Iberian Peninsula in the Pyrenees mountainous ridge. It is located near Canillo village (Figure 1B) and on the east margin of the Valira d’Orient river. The landslide involves a mass of approximately 300Mm3 and with an extension of about 2.7Km2 [Corominas et al., 2013]. This landslide has been formed by several destabilizations since the Upper Pleistocene leading to accumulations of large sliding deposits. It was first described by Corominas and Alonso [1984] and after by Soutadé [1988], who postulated that the mass slid toward the valley after the Valira d’Orient glacier melted between 20.000 and 10.000 BP.

After these initial studies, Corominas [1990] described the slope stability induced by glaciers in El Forn and Encampanada. Santacana [1994] studied in depth the big landslide of El Forn and the lateral expansion of Encampadana, which resulted from the melting of the Valira d’Orient glacier in the main valley, between 16.000 and 13.000 BP or lower. The latter study also divides the El Forn landslide formation process into three main episodes: the first is a complex rotational/translational movement with flow at the toe at the Occidental margin of the El Forn area, which could have collapsed the main valley; the second is also a rotational/translational complex movement that collapsed the valley thereby creating a lake; and the third is a rotational movement that fossilized some parts of the rotational shear bands of the second movement. Santacana [1994] also pointed out that there are recent movements at the toe of the landslide.

More recently, Planas et al. [2011] studies exhaustively the movements produced in the El Forn landslide following the work of Santacana [1994], and with the support of the data of drills performed in the landslide, work field, and new radiocarbon dating. Planas et al. [2011] concluded that the El Forn landslide has been subject to at least three large movements at different time periods, however not all of them are from the postglacial era, since there are at least two big episodes interspersed by small progress of the glacier over the sliding mass. The most recent work performed for this landslide has been Jaqués [2014] who did a review of the El Forn landslide describing the background of the landslide (until that time) and proposing a new geological model of the slide with a new reactivation sequence, including a slope stability analysis. Furthermore, Jaqués [2015] performed a numerical analysis using finite element method to reproduce the behavior of the large slide and the most active areas of the sliding mass.

The two main risks of the landslide being active are: the area of the landslide is constructed and has agricultural lands, and the landslide is sliding towards the Canillo village (North-West direction). Due to the risks, and the presence of indicators that the landslide is active (i.e., fractures and subsidence in the roads, fissures on the walls of buildings and retaining walls, etc.) the Andorra Government started in 2000 the realization of risks and geologic maps [Altimir et al., 2001; Euroconsult, 2002]. Afterwards, joining with Euroconsult company, started in 2007 a drill campaign along the extension of the landslide [Euroconsult, 2007]. This campaign had the goal of monitoring the movement of the landslide and the groundwater table in order to understand the danger of the landslide to society and infrastructures. There have been also studies of radar and differential interferometry techniques (GB-SAR and DInSAR) on the sliding mass to follow its movement [Corominas et al., 2015].

The data provided by the instrumentation of the drills as extensometers and inclinometers [Euroconsult, 2008 to 2018], radar and differential interferometry techniques [Corominas et al., 2013; Corominas et al., 2015] shows that inside the El Forn landslide there are three areas that slide faster: Cal Ponet – Cal Borronet (Center-North), Clots Fondos (North-East), and Prats (at the toe of the slide, East)  with velocities around 4cm/year. Figure 1C shows the displacement of the Cal Ponet-Cal Borronet lobe between April and June of 2017 with a movement of over 2 cm [Euroconsult, 2017]. Previous studies have suggested that triggering factors of the landslide are: 1) rise of groundwater level, and 2) erosion of the toe of the landslide due to the Valira d’Orient river [Corominas et al., 2015].

In this study we will focus on one of the three areas that are sliding faster: Cal Ponet – Cal Borronet. This area has been described and delimited by Torrebadella et al. [2009] as one of the fastest creeping lobes of the El Forn landslide involving a mass of 0.97 Mm3. The authors presented the results of the group of lithologies of the landslide from the core samples of the drills performed between 2007 and 2009, which are divided by three main groups: colluvium, high fractured and altered shales, and shales of the bed rock. Furthermore, they show the results of the geotechnical parameters of each group of geological units for each drill performed and the instrumentation installed inside the boreholes. By the time the authors did the report, they already had field data to describe the areas of the landslide that were creeping faster at that time, defining the Cal Ponet – Cal Borronet lobe.

Focusing now on the Cal Ponet – Cal Borronet lobe, the first drill was performed in 2007 with core sample recovery – which is the material that has been used to perform this study - and instrumented with piezometers and inclinometers. Due to the movement of the landslide and, thus, the loss of some instrumentation and the borehole below the shear band, the Andorra Government has performed 4 more drills next to the initial to continue monitoring its movement and groundwater. The last drill and installation of instrumentation were realized by the Andorra Government and the Polytechnic University of Catalonia. The latter was performed in September of 2018, did not have core sample recovery; however, it was instrumented with three piezometers at different depths (one below the shear band and two above it), a wire extensometer [Corominas et al., 2000] (with the base at 40m depth) and a thermometer in the shear band at 29m depth.

Figure 1. A) Map of Andorra country with a red circle indicating the location of the area of study. B) Satellite image (Google Earth) presenting: El Forn landslide (in red), red arrow indicating the direction of the sliding movement, Cal Ponet-Cal Borronet lobe (in purple) with the location of the S10 drill (white marker). C) Displacement data of the S10 drill from April to June 2017 [Euroconsult, 2017].

## Geological and Geomorphological setting

From a geological point of view, the Andorra country is located in the Central-Eastern part of the Pyrenean Axial Zone. Therefore, due to the orogeny of the Pyrenees, the structural morphology of the Andorra Principate is complex including anticlines, synclines and faults [see Hartevelt, 1970; Zwart, 1979; Barnolas and Chiron, 1996] formed by Paleozoic materials (Cambrian, Ordovician Silurian and Devonian) that have been intensely deformed in a complex process during long periods [Clariana, 2004]. These rocks are associated to detachments and diapirs causing softness and incompetent behavior [Zwart, 1979]. The Devonian materials are defined in this area as lutites, calcareous lutites interbedded with thin limestones layers (Rueda Fm.) to massive (up to 60m thick) limestones and marly limestones (Castanesa Fm.), and green to yellow marly limestones (Manyanet Fm.).

Figure 2 presents the area of interest in this study, which is the El Forn landslide (defined as a discordant contact by a black line) that is formed by Paleozoic materials (Cambro-Ordovician to Mid Devonian), and the Cal Ponet-Cal Borronet lobe (defined as a discordant contact by a blue line) formed by Silurian materials. These Paleozoic materials, specifically the Silurian ones are found as limestone and shale (in purple) from the Western to the South-Eastern areas of the landslide. Moreover, there are 3 formations of the Devonian at the East and North-East part of the landslide: 1) marly limestones and harsh shales of the Lower-Mid Devonian (in green), and two intrusive contacts of the lower Devonian as 2) massive limestones (in blue) and 3) alternations of limestones and brown shales (in geen-yellow). In the South-Eastern part of the landslide there are series of rhythmic sandy shales (in white) of the Cambro-Ordovician [see Clariana, 2004 for a detailed stratigraphy].

The structural geology of these materials is arranged in East-West or ENE-WSW direction. The pre-carboniferous rocks are defined in anticline and syncline domes with a kilometric extension and with faults and thrust at the boundaries of the domes oriented in the same direction as the domes. The El Forn landslide area is located in the Tor-Casamanya syncline, where there are present the Devonian materials in the center and Silurian and Cambro-Ordovician materials at the sides [Clariana, 2004].

The geomorphology of the El Forn landslide area can be described as the large slide (in black) including the intrusive contacts in the East part of the landslide (yellow-ish lines as contacts between Lower-Devonian formations and Lower-Mid Devonian formation and Silurian). The slide is surrounded by a large area of not appreciable metamorphism in the south, and in the north-western part of the landslide there is another intrusive contact of Lower Devonian (green-yellow).

Figure 2. Geological map of the area of study [Clariana, 2007].

# Methodology

## Samples used in the study

The samples left from the 2007 drill from the Cal Ponet – Cal Borronet lobe (Figure 3) were ones in the box that the shear band is in (from 25 to 30m depth). From Figure 3 and the data (piezometers) from Euroconsult, the samples are fully saturated. It is known that the shear band is between 28 and 31m depth due to the data of the extensometers and inclinometers, provided by Euroconsult, where the vertical profile of displacement (Figure 1C) shows that the mass of rock is moving as a rigid-block (translational/rotational).

Figure 3. The only box preserved with the core samples of the S10 drill performed in 2007, including the shear band of the lobe.

To perform the different tests to characterize the shear band, we have separated the samples at each half a meter, thus, having 9 samples to perform every test. Furthermore, we took shales core samples of the bedrock (174 to 175m depth) from another drill performed in the landslide to compare the “pure” shales of the bedrock with the “altered” shales of the shear band.

## Sampling

The samples studied in XRPD needed to be crushed in an agate mortar until the rock reduces its particle size below 40 m, ending up with a texture of very fine powder that does not scrape between the fingers. To study the samples in SEM-EDS we have done polished thin sections. The procedure of sample preparation of thin section includes the cut of a 1000 mm3cubical or prismatic sample from a cylindrical core of undisturbed material retrieved at a depth between 25m to 30m in this case. The cubical sample is first included in epoxy resin and then stuck on a glass sample holder -on one side-, to cut a slice of 50 microns thick on which SEM observations will be performed.  Finally, a metallographic polishing -under dry conditions-, is performed to obtain the flat surface needed for EDS analyses.

To perform the tests of the samples in the MicroCT, the samples are the original ones, obtained from the core samples. We have chosen small, representative, rock pieces of the order of cm to perform each test. Multiple tests have been performed to ensure representativeness of the samples and repeatability of the results.

## Analytical Techniques

### XRPD (X-Ray Powder Diffraction)

X-ray Powder Diffraction (XRPD) data were collected with a Panalytical X’Pert PRO MPD X-ray diffractometer with monochromatized incident Cu Kα1 radiation at 45 kV and 40 mA, equipped with a PS detector with amplitude of 2.113°. Patterns were obtained by scanning random powders from 4° to 80° (2) on samples powder or on oriented mounts. Datasets were obtained using a scan time of 50 seconds at a step size of 0.017° (2) and variable automatic divergence slit. Identification of the minerals was achieved by comparison of the XRPD diffractogram with the ICDD data base (2007 release) using the Diffrac plus Evaluation software [Bruker, 2007]. Quantitative mineral phase analyses were obtained by full refinement profile using XRPD. The software used was TOPAS V4.2 (2009) [Coelho, 2000].

### SEM-EDS (Scanning Electron Microscope with Energy Dispersive X-Ray Spectroscopy)

The morphology and microtextural features of the studied samples were examined on polished thin sections with a Nikon Eclipse LV100 POL microscope and an ESEM Quanta 200 FEI, XTE 325/D8395 scanning electron microscope with energy dispersive X-ray spectroscopy (SEM-EDS).

### MicroCT (Micro X-Ray Computed Tomography Scanner)

The porosity of the samples used in this study was examined on small sizes of original core rock (order of cm) with a Nikon XTH 225 ST, which is a high-resolution X-Ray computed tomography scanner. With this scanner, 2-D and 3-D images of the interior and the exterior of the sample are obtained by the projection of X-Ray beam through the sample and the sample’s interaction with the beam via radiography/x-ray image. The X-Ray source used in the tests performed is 225 kV UltraFocus Reflection Based Signal and Tungsten Target (Spot Size 3m) with a Max Power of 200 W. The detector used is the Perkin Elmer 1620 AN3 CS CT. The X-Ray filter used in these tests is a 0.5 mm thick copper filter. The post-processing of the 2D and 3D images and its reconstruction has been performed in Avizo Software [TJP, 2015].

The technical features of the images performed by the MicroCT Scanner are resolution (voxel size) between 0.0265 and 0.0443 mm, detector 2000x2000 pixels with each pixel spaced at 200 m, 2500 projections, angle steps of 0.144 degrees, and X-Ray energy of 190 kV.

## Plasticity Index

The plasticity index has been obtained for the studied samples. For each sample, we have performed three tests in order to obtain reliable results. First, has been performed a Plastic Limit test, and second a Liquid Limit test. The geological material used for the tests has been the finer size of the core samples mixed with small grains (mm). To perform the liquid limit test, it has been used the Casagrande liquid limit device.

# Results

## XRPD

Nine samples have been tested in this study. Each one representing half a meter of the stratigraphic column (shear band depth), and the bedrock shales as the unaltered and initial material of the shear band. In total, there are 6 mineral phases found in the samples: quartz [SiO2], muscovite [K2Al4Si6Al2O20(OH,F)4], chlorite chamosite [(Mg,Al,Fe)12(Si,Al)8O20(OH)16], paragonite [(K,Na)2Al4Si6Al2O20(OH,F)4], calcite [CaCO3], and plagioclase (albite) [NaAlSi3O8].

Note: the paragonite found in the XRPD test it is not pure paragonite mineral, NaAl2(Si3Al)O10(OH)2 [Mineral Data Publishing, 2001], it is an altered muscovite rich in sodium (as it is shown in its chemical formula in the paragraph above). However, in this paper we will call it Paragonite (see section 5.1 for in depth reasoning).

The percentage in weight of the identified mineral phases has been calculated by the Rietveld method [Rietveld, 1969] and using TOPAS V4.2. software.

Figure 4 presents the results of the diffraction for each sample. The 25.5-26m is mainly composed by calcite (52 wt%), quartz (12 wt%), muscovite (25 wt%) and chlorite chamosite (11 wt%), it is a sample that its mineral composition is very different compared to all the other studied samples. Samples 26.5-27m and 27-27.5m are formed by quartz (between 35 and 28 wt%), muscovite (between 55 and 52 wt%), chlorite chamosite (between 16 and 13 wt%) and with indicators of calcite (≤ 1 wt%). There is a group of five samples that present a composition and proportion of identified minerals very similar, which are: 27.5-28m, 28-28.5m, 28.5-29m, 29-29.5m, and 29.5-30m, which all of them form the shear band. In these samples a muscovite rich in Na has been identified, a mineral called paragonite. In all the samples the presence of quartz varies between 24 and 21 wt%, muscovite between 59 and 53 wt%, paragonite between 22 and 14 wt%, and chlorite chamosite between 6 and 3%. The sample corresponding to the Silurian shales, which is the bedrock material, is very rich in muscovite (80 wt%), plagioclase (albite 12 wt%), quartz (8 wt%), and only 1 wt% of chlorite chamosite. The latter is the only sample where has been detected the presence of a feldspar (albite).

Figure 4. Quantitative results of

each sample obtained by the XRPD tests performed.

## SEM-EDS

The results of all the samples studied revealed four different textures. Thus, we are going to present the results of the samples textures by groups.

The first textural group is defined by the sample 27-27.5 m. This sample is a black-gray shale with several small fractures filled by quartz (Figure 5A and 5B), between phyllosilicates (muscovite, paragonite and chlorite). Muscovite crystals are seen with a certain orientation, but there is not a defined direction in the whole sample and shows a fluid texture of layers of muscovite. Quartz grains are found in the sub-horizontal fractures with different sizes of thickness between the phyllosilicates. There is also a presence of accessory minerals as apatite and chloritoid (which is very rich in REE – Ce, La and Nb -). In Figure 5C can be seen that there is a material with phyllosilicates very poor crystalline at the boundaries of the quartz veins which could indicate the reaction that is produced between the circulating fluids and the rock.

The second textural group is defined by the sample 27.5-28 m. Visually, this sample is completely different from the others but the SEM-EDS images of the fragments are identical at the samples of the first group. It is composed by large angular fragments of black-gray shales with irregular shape of different sizes that are surrounded by a thinner matrix (Figure 5D). The matrix is composed by small crystals of phyllosilicates (muscovite, paragonite and chlorite) that are not attached; thus, these big clasts of shales are not matrix supported, as can be seen in Figure 5F. The texture of a fragment of this sample (Figure 5E) is identical to the texture of the 27-27.5 m sample (Figure 5B).

The third textural group is defined by samples 28-28.5 m, 28.5-29 m, and 29.5-30 m. Visually, these samples are black-gray shale with several small and large fractures filled by quartz, between phyllosilicates (muscovite, paragonite and chlorite). Muscovite crystals are seen with a certain orientation and direction in some parts of the samples (especially in the folds, Figure 5G), but in the other areas of the samples there is not a defined direction that shows a fluid texture of layers of muscovite. Quartz grains are found between the phyllosilicates following the orientation of folds (Figure 5H). There are also present in the samples accessory minerals as rutile (titanium oxide) and hematite (ferrous oxide). In Figure 5I can be seen a detailed image of the crystals of muscovite and chlorite.

The fourth textural group is defined by sample 29-29.5 m, where has been found the shear band. Visually, is a very homogeneous (texturally) sample very dark (black shales) and large and oriented fractures filled by quartz. In the SEM it can be seen a well-defined orientation and direction of the phyllosilicates (muscovite, paragonite and chlorite) in the entire sample (Figure 5J). There are also thick fractures that are not filled by quartz, where can be seen broken phyllosilicates crystals in the direction of the fracture (Figure 5K). The fractures are formed parallel to the phyllosilicates. In this case, the contacts between the phyllosilicates are bond in their large plane/sheet (Figure 5L). The texture of this fourth textural group is different from all the other textural groups, as there are no developed folds and can be seen that the phyllosilicates are all arranged following the same direction.

Figure 5. SEM-EDS images. First textural group: A) Backscattered electron image of sample 27-27.5 m. In this image the main mineralogy is mainly quartz and phyllosilicates. Image shows the thickness variability of quartz veins (see yellow lines). There are present accessory minerals of elevate reflectivity, apatite (very light gray color) and possibly another mineral rich in REE (vivid white color). B) Backscattered electron image of sample 27-27.5 m: the yellow lines show a thin layer of phyllosilicates between blocks of quartz. C)  Backscattered electron image of sample 27-27.5 m. At the top and bottom parts of the image are large blocks of quartz (yellow lines), and in between two kind of phyllosilicates (divided by a red line): low crystalline material form by phyllosilicates, and higher crystalline phyllosilicates. Second textural group: D) Backscattered electron image of sample 27.5-28 m. Fragments of shales of different sizes surrounded by small crystals of quartz and phyllosilicates without cement. E) Backscattered electron image of sample 27.5-28 m. Detail of a fragment of shale where can be observed the texture of the minerals. In this case, there are no visible veins of quartz. The crystals of quartz are fractured and occupy the positions left by the crystals of muscovite and chlorite, which present deformation and form folds surrounding the quartz. The orientation of the phyllosilicates is variable. F) Backscattered electron image of sample 27.5-28 m. Zoom of the matrix, composed by big crystals (23m) of quartz that are slightly fractured, and small crystals of muscovite as rectangular sections. There is a rutile crystal (vivid white color). Third textural group: G) Backscattered electron image of sample 28.5-29 m. In this case the folds are totally visible. These folds (purple line) are formed by the deformation produced due to shear. In general, the dimensions of the folds are bigger, due to sinuosity of the fold is higher. The phyllosilicates are more plastic and deform easily forming the folds. However, quartz is not plastic and does not orient and occupies more stable positions as inside the folds. H) Backscattered electron image of sample 28.5-29 m. Detail of the orientation of the phyllosilicates in a syncline (purple line). I) Backscattered electron image of sample 28.5-29 m.  Zoom of the crystals of quartz, muscovite, chlorite and an intergrowth of chlorite and muscovite. Fourth textural group: J) Backscattered electron image of sample 29-29.5 m. Mainly phyllosilicates that are oriented. The orientation is clear following the white crystals (chlorite). K) Backscattered electron image of sample 29-29.5 m. In the left there is a large vein of quartz, and the rest are mainly phyllosilicates that are totally oriented (see the crystal boundaries). There is a large fracture (black) in the middle of the image where can be seen that the phyllosilicates follow the boundaries of the fracture. L) Backscattered electron image of sample 29-29.5 m. Oriented phyllosilicates: Chlorite (white), muscovite (grey), paragonite (dark grey) and intergrowths muscovite-paragonite.

## MicroCT

Six zones/samples have been tested to study the porosity by MicroCT Scanning. The results shown in Figure 6 are for three samples, one of each mechanical group (slightly different from the textural groups). We will also show the results of all the samples to see the evolution of porosity along the shear band.

The first mechanical group is defined by the samples 27-27.5 m, 27.5-28 m, and 28-28.5 m. These samples present a volume fraction of the matrix of 0.226, 0.2, and 0.23 and a volume fraction of porosity of 0.0029, 0.0027, and 0.0027, respectively. Thus, with a percentage of pores in the samples of 1.27%, 1.37%, and 1.15%.  In sample 27-27.5m (Figure 8) can be seen visually that the top part of the sample has less porosity, and in the bottom, there is more porosity without a defined form and orientation. Furthermore, can be seen a fracture at the bottom of the sample. Sample 27.5-28m presents a defined orientation and direction of small pores and some large fractures, which follows the direction of the shear movement of the landslide.

The second mechanical group is defined by samples 28.5-29 m and 29.5-30 m. These two samples present a volume fraction of the matrix of 0.14 and 0.22, and a volume fraction of porosity of 0.00021 and 0.00019, thus, with a percentage of pores in the sample of 0.14% and 0.087%, respectively. This mechanical group presents some fractures in the SW part which are oriented and with the same direction. There are also a few big pores in the NE part of the sample that are not oriented or with a defined shape. The sample 29.5-30 m is presented in Figure 6 where can be seen some very thin and large fractures NW-SE direction across the entire sample. Furthermore, this sample presents small pores that are aligned and organized in bands, these bands have all the same direction as the thin fractures.

The third mechanical group is defined by sample 29-29.5 m, where has been found the shear band (defined visually in the MicroCT as a large fracture of porosity, the highest percentage of porosity of all the samples). This sample presents a volume fraction of the matrix of 0.14 and a volume fraction of porosity of 0.0033, thus, with a percentage of pores in the sample of 2.16%. Visually, this sample presents at its center a large fracture - that crosses the entire sample (Figure 6) – partially filled but also with some large pores interconnected (where the water can flow through them), with a direction (E-W). Above and below the large fracture can be seen a few thin fractures (some – in the bottom of the sample - with larger pores) all of them oriented in the same direction as the shear band).

Figure 6. MicroCT 2-D section images, and 3-D reconstruction. The samples shown in this figure are one of each mechanical group. In the images of the 2-D section, the black color represents the voids/porosity, which is represented in the 3-D reconstructions as the dark color (dark grey or dark green). In the samples 27.5-28 m and 29-29.5m is clear in the 3-D reconstruction the orientation of the pores. However, in the sample 29.5-30m the porosity in the 3-D reconstruction is poorly defined as the pores are very small (due to the low porosity of the sample). Thus, for this last sample the 2-D section image shows clear the orientation of the pores (in black).

## Plasticity Index

In this section we present the plasticity index results in relative values (all the results obtained will be divided by the value obtained for the first sample - 27-27.5m-). This is due to: 1) the values obtained in the Liquid Limit and Plastic Limit tests usually depend on the user, as each user adds different quantities of water in the sample, and 2) when the drill was performed, the company obtained a plasticity index value for the background value of the sample 27.15-27.30 m of 8.7 and now we obtained for the sample 27-27.5 m a plasticity value of 3.96. This difference of plasticity index value could also be due to the dry state of the samples, and its preservation after 12 years (when the drill was performed).

The results of the plasticity index reveal the same behavior as the porosity results (Figure 8). These results show three distinct values which can be identified as three mechanical groups (same mechanical groups as MicroCT). The first mechanical group is composed by samples 27-27.5 m, 27.5-28 m, and 28-28.5 m, with values of relative plasticity index of 1, 0.93, and 0.97, respectively. The second mechanical group is composed by samples 28.5-29 m and 29.5-30m, with relative plasticity values of 0.67 and 0.62, respectively. The samples of the second mechanical group are located above and below the shear band, with lower values of relative plasticity index compared to the values of the first mechanical group (over 30% lower). And, finally, the third mechanical group, which is composed by sample 29-29.5 m - the shear band-, with a value of relative plasticity index of 1.95. Thus, this third mechanical group has a very high value of relative plasticity index compared to the rest of the groups.

# Synthesis of the results

## Chemo-mechanical alteration of shales inside the shear band

With the x-ray diffraction test we have seen a change in mineralogy of the shear band’s black shales compared to the bedrock black shales (Figure 7). The original composition of the black shales has been obtained from the bedrock shales, which are formed by quartz, muscovite, chlorite, and albite. However, in the “altered” shales, which are considered in this study as the shear band shales, its composition is quartz, muscovite, chlorite, and paragonite. The presence of paragonite and the non-appearance of the albite, indicates that chemical and mechanical processes have been involved [Livi et al., 1990; Li et al., 1994; Livi et al., 1997; Arkai et al., 2008]. Paragonite has been found in Central and SE- Europe and the previous authors, among others, have been studied the cause of the rare appearance of this mineral in metamorphic rocks, which generally forms in areas that have undergone by high pressures and low temperatures [Frey, 1978, 1987a; Arkai, 2008]. Specifically, in Silurian to Jurassic shales (as the ones that we study), the appearance of paragonite is extremely rare [Arkai, 1983; Arkai and Kovacs, 1986; Arkai et al., 1995; Arkai, 2008].

The paragonite found in this study is related with the muscovite and presents an intermediate composition between muscovite (K2Al4Si6Al2O20(OH,F)4) and paragonite (NaAl2(Si3Al)O10(OH)2). Its result is an intermediate mineral which is a muscovite rich in sodium defined in this study as paragonite ((K,Na)2Al4Si6Al2O20(OH,F)4). Usually paragonite can be found as inner growths inside the muscovite [Arkai et al., 2008; Gioretti et al., 2003; Jiang and Peacor, 1993].

Different mechanisms that can be involved in this chemo-mechanic alteration of the black shales. Some studies consider that as the albite (NaAlSi3O8) is a mineral rich in sodium, hydrothermal alteration can contribute Na to the fluids that circulate through the rock and trigger the formation of paragonite [Chatterjee, 1973; Frey, 1969, 1978, 1987a; Arkai, 2008; Susmita Gupta and Fareeduddin, 2013]. However, considering the percentage of albite in the Silurian shales (bedrock sample) and the percentage of paragonite in the altered shales (shear band sample), we can see that the percentage of these two minerals at each sample is almost the same. Moreover, it is important to overlay that the paragonite in our samples has been seen by SEM as an intergrowth between muscovite particles. Thus, there is also another mechanism that could had happen in the samples to explain the alteration. Gioretti et al., [2003] studied the processes that lead to the appearance of an intermediate muscovite rich in sodium as an intergrowth between muscovite minerals. He described that the result of an altered muscovite rich in sodium is formed by an incomplete reaction of muscovite towards paragonite, ending up with an intermediate mineral phase. Therefore, in our case study, we could consider the possibility of hydrothermal alteration of both, albite and muscovite, as both minerals are in contact with each other, especially if the albite grains are fully surrounded by muscovite. This could explain that there is a process involving the groundwater and high pressures that trigger the dissolution of the albite, and as albite is in contact with muscovite, creating an intermediate phase between muscovite and paragonite.

The percentage of quartz is also very different. In the five samples of the shear band, the percentage of quartz varies from 24 to 21 wt%, while in the bedrock Silurian shales its of 8 wt%. This difference of amount of quartz can be interpreted as the formation of fractures inside the rock that facilitate the hydrothermal fluid transport and the subsequent precipitation of quartz inside the fractures. This interpretation is confirmed by the presence of fractures filled by quartz observed visually in the samples and in the SEM-EDS images.

Figure 7. Comparative of the mineralogy between the Silurian shales and the shear band black shales. The main difference between them is the presence of 12% of albite in the Silurian shales and, in another hand, the presence of 14% of altered muscovite (paragonite) in the shear band black shales. Furthermore, the increase of chlorite from 1% in Silurian shales to 6% in shear band black shales. Also, the increase of quartz in the shear band (due filling of fractures created due to high stresses), and the over 20% decrease of muscovite in the shear band black shales.

## Mineralogical and textural evolution of the shear band

There have been differentiated 4 groups regarding its texture joint with its mineralogy content, called textural group: 1) the first textural group is defined by the sample 27-27.5 m. This sample is composed by 35% of quartz, 52% muscovite, 13% of chlorite, and 1% of calcite (Figure 8). It is the only sample of the shear band that does not have any paragonite, and the only that has a small content of calcite. Furthermore, is the sample that has a higher content of quartz (35% compared to 21-24%) and chlorite (13% compared to 3-6%) in the shear band. Texturally, this sample presents at small scale a certain orientation, but not a defined direction at a larger scale. 2) the second textural group is defined by the sample 27.5-28 m. This sample is composed by 21% of quartz, 53% of muscovite, 3% of chlorite, and 21.81% of paragonite (Figure 8). Thus, this sample is the one that has a higher content of paragonite. The texture of this sample is very different from the rest of the samples of the shear band. This sample presents large angular grains (mm) composed by quartz and phyllosilicates, and at their bonds there are present small fragments of phyllosilicates, but not a dense matrix. Thus, the sample is clasts supported. The texture inside the clasts is exactly the same as the sample of the first textural group. Therefore, this new texture could imply, as the groundwater table is at this same depth, that the force of the flow has broken the initial texture (first textural group) into clasts and, afterwards, the fluid transport has filled the bonds of the clasts with phyllosilicates. 3) the third textural group is defined by samples 28-28.5 m, 28.5-29 m, and 29.5-30 m. These samples have the following composition: 24%, 21%, and 24% of quartz; 55%, 59%, and 24% of muscovite; 6%, 5%, and 5% of chlorite; and 15.13%, 15.32%, and 17.5% of paragonite, respectively (Figure 8). The percentage of composition of each mineral of the three samples is very similar. In general, the texture of the three samples is very similar as well, where the phyllosilicate crystals have a certain orientation and direction, due to the presence of folds. These folds are formed due to high stresses that are affecting these samples because of their location, above and below the shear band. 4) the fourth textural group is defined by sample 29-29.5 m. This sample is composed by 24% of quartz, 56% of muscovite, 6% of chlorite, and 14.16% of paragonite (Figure 8). Thus, regarding its composition, this sample could be included inside the third textural group, however, due to its texture, this sample requires to be included into an additional textural group. The texture of this sample is different from the others, as there are no folds present, and the phyllosilicates are totally aligned and oriented in one direction. The phyllosilicates are attached by their large plane-plane or sheet-sheet of the crystals, which is the weakest bond between phyllosilicates. This kind of bond attaching the phyllosilicates and its orientation facilitates the friction between the crystals and the formation of fractures in

the same orientation and direction. The complete orientation of the crystals along the entire sample identifies it as the shear band, due to mechanical stresses because of the friction, which orients the crystals. The bonds between phyllosilicates in presence of water, helps the friction between them, reducing its resistance in front of displacement. The presence of paragonite in the sample also helps the friction due to is a mineral with more content in Na (i.e., more soluble).

## Mechanical evolution of the shear band

There have been differentiated 3 groups regarding its porosity joint with its plasticity limit, thus its mechanical properties:  1) the first mechanical group is defined by the samples 27-27.5 m, 27.5-28 m, and 28-28.5 m. This first group present a porosity between 1.15 and 1.27% (Figure 8). The small pores are, generally, placed along the entire sample with a defined direction (probably the shear direction), but they tend to be mainly in the bottom of the samples. The porosity is not connected as the pores are isolated. The fractures inside the samples follow the same direction as the pores, which could be an indicative of the shear movement. The values of the plasticity index in these samples are very similar, varying between 0.93 to 1. Thus, these three samples – located outside the mechanical stress area of the shear band - have the same properties regarding their porosity and plasticity index. 2) the second mechanical group is defined by samples 28.5-29 m and 29.5-30 m. These two samples are located right above and below the shear band, and they present the lower porosity values of this study, between 0.087% and 0.14% (Figure 8). In general, the pores present in these samples are very small, and not connected between them, but they follow a defined orientation and direction – the shear direction – which is the same as the fractures formed in these samples. The plasticity values in these samples are also very similar, with values between 0.62 and 0.67 (Figure 8), which are the lowest ones compared to the rest of the samples studied. Thus, in the light of these results, can be considered that right above and right below the shear band there is a strong mechanical behavior that produces a reduction in the porosity and plasticity of the samples due to the reorganization of the mineralogy because of large deformation and stresses. 3) the third mechanical group is defined by sample 29-29.5 m. This sample has been identified as the shear band of the deep-seated landslide. The study of porosity revealed a thick fracture present along the entire sample, which indicates a path of water flow. The porosity of this sample is the highest one compared to the rest of the samples studied, with a value of 2.16% (Figure 8), where all the pores are concentrated and connected forming a thick and large fracture. Furthermore, the plasticity index in this sample is the highest one compared to the rest of the samples studied, with a value of 1.95 (Figure 8). This high plasticity it is not related its mineralogical composition – as the rest of the sample have the same composition and percentage values of the minerals – but to its fabric/texture [Chester and Logan, 1987; Colletini et al., 2009; Tchalenko, 1968; Maltman, 1987]. This high plasticity can be explained by the way that the phyllosilicates are perfectly oriented with the shearing direction. As phyllosilicates with oriented fabric are notably weaker especially in the presence of water, this is typically reflected in their plastic behavior by increasing the plasticity index [Labaume et al., 1997].

[Figure 8 should be positioned here]

# Discussion and Conclusions

This paper presents the results of the study -at micro-scale- of the shear band of the Cal Ponet - Cal Borronet lobe, which is located inside the large El Forn landslide. This lobe is an active deep-seated landslide, as it creeps as a rigid block. As the shear band is the weakest part of a landslide, this paper has presented the results and a correlation between them of the tests performed on the material that forms the shear band (core samples) and the bedrock black shales. These tests are: X-Ray diffraction to study the percentage of the mineralogy, SEM to study the mineralogical texture, MicroCT to study the porosity, and plasticity index to study the plastic behavior of the material.

The shear band of this case study is formed by black shales, which are very rich in phyllosilicates. With the tests performed in the samples of the shear band and its results, it can be seen that there are two kinds of deformation/alteration of the material: chemical and mechanical alterations. The chemical alteration can be seen as the difference of mineralogical composition between the black-shales located in the shear band (considered as the altered material) and the black-shales located in the bedrock (considered as the initial material). The main difference between the altered and the initial black shales is that in the shear band there is a presence of paragonite (Figure 7), which is an altered muscovite rich in Na, and that in the initial shales instead of paragonite there is albite, which is a plagioclase rich in Na. Thus, this chemical – and mechanical- alteration of the muscovite/albite inside the shear band has been evolved due to hydrothermal alteration and high pressures. The partial dissolution of the albite by hydrothermal flow joined with the surrounding muscovite, and at high pressures, produces an intermediate muscovite rich in Na, called paragonite.

The mechanical alteration due to the localized shear movement (active landslide) is represented in the results of the textures of the samples. These results show an evolution of the arrangement of the minerals, where outside the shear band there is no orientation nor direction defined of the mineralogy, but closer to the shear band the minerals are rearranged and more organized forming folds. Finally, inside the shear band it can be seen a clear and defined orientation of the minerals in one direction (same direction as the shear movement), as well as the fractures formed in the material (Figure 9). The mechanical alteration of the material can also be explained by the results obtained from the porosity and plasticity index tests. It can be seen that the porosity and plastic behavior outside the shear band -along a meter and a half- is the same, with a continuous porosity localized in small pores not connected but oriented in the flux direction, and with the same relative value of plasticity index. However, the samples from above and below the shear band present a completely different behavior in which both, the porosity and plasticity index decrease sharply their values. Those lower values can be explained due to a rearrangement of the minerals forced by the shear movement nearby. This rearrangement of the mineralogy along with high pressure of the shear movement reduce its pores and, because of the folds that are present due to large stresses, the minerals are not bonded in a way that the shear deformation is facilitated, thus, decreasing its plastic behavior. Finally, the material that is located in the shear band has a high porosity value, which is localized in a high porosity zone that follows the shear movement. The material that surrounds the shear band also has a very high value of plasticity index, compare to the rest of the samples studied. The plastic behavior in this area can be explained by the texture, which indicates the alignment of the phyllosilicates bonded by their large plane-plane, which is the weakest bond between phyllosilicates crystals, and this bond induces its plasticity index. Furthermore, the presence of paragonite in the sample (phyllosilicate rich in Na) helps the friction between the particles due to paragonite is a more soluble mineral than the other phyllosilicates present in the samples (muscovite and chlorite).

Figure 9. Profile of the Cal Ponet-Cal Borronet lobe with a sketch of the orientation of the mineralogy inside (maximum thickness of the layer: 0.5m) and outside the shear band (maximum thickness of the layer: 2m). Note: As the samples tested represent 0.5m of the stratigraphic column, there is no scale for such sketch. However, it represents in dimensionless form the evolution of the orientation of the minerals along the stratigraphic column (inside and nearby the shear band).

In conclusion, the micro scale study performed in this paper can explain the chemical and mechanical alterations happening on a clayey shear band of a deep-seated landslide. With the results of the tests performed on the core samples of the shear band of El Forn landslide and outside the shear band, we have been able to understand the mechanisms that occurred and occurring in the weakest part of a landslide (i.e., the shear band). This understanding is based on correlating the results obtained outside and inside the shear band at each test, as well as the results of each tests, as we have seen that each property is directly dependent on another.

# Acknowledgements

The thin sections were made at the "Servei de Làmina prima" from the Faculty of Earth Sciences in Barcelona University. The XRPD and SEM-EDS analyses were made and performed at the Scientific and Technological Centers of the University of Barcelona (CCiTUB). The MicroCT tests were carried out at the Duke University Shared Materials Instrumentation Facility (SMIF), a member of the North Carolina Research Triangle Nanotechnology Network (RTNN), which is supported by the National Science Foundation (Grant ECCS-1542015) as part of the National Nanotechnology Coordinated Infrastructure (NNCI). The liquid and plastic limit tests were carried out at the Soil Mechanics laboratory at Duke University. The authors acknowledge the help of the laboratory technician Michael Blagg in performing the tests.

# REFERENCES

Altimir, J., Copons, R., Amigó, J., Corominas, J., Torrebadella, J., Vilaplana, J. M. (2001). Zonificació del territori segons el grau de perillositat d’esllavissades al Principat d’Andorra. Primeres Jornades del CRECIT (IEA), La gestió dels riscos naturals: 119-132.

Anderson, D. L. (1980). An earthquake induced heat mechanism to explain the loss of strength of large rock and earth slides, in Proceedings of the International Conference on Engineering for Protection from Natural Disasters, pp. 569-580, John Wiley, New York.

Árkai, P. (1983). Very low- and low-grade Alpine regional metamorphism of the Paleozoic and Mesozoic formations of the Bükkium, NE-Hungary. Acta Geologica Hungarica, 26, pp. 83–101.

Árkai, P., S. Kovács (1986). Diagenesis and regional metamorphism of the Mesozoic of Aggtelek–Rudabánya Mountains (Northeast Hungary). Acta Geologica Hungarica, 29, pp. 349–373.

Árkai, P., K. Balogh, I. Dunkl (1995). Timing of low-temperature metamorphism and cooling of the Paleozoic and Mesozoic formations of the Bükkium, innermost western Carpathians, Hungary. Geologische Rundschau, 84, pp. 334–344.

Árkai, P., K. J. T. Livi, P. Horváth (2008). Dioctahedral mixed K-Na-micas and paragonite in diagenetic to low-temperature metamorphic terrains: bulk rock chemical, thermodynamic and textural constraints. Central European Geology, Vol. 61/4, pp. 283-314.

Barnolas, A., Chiron, J. C. (1996). Synthèse Géologique et Géophysique des Pyréenées, Volume 1: Introduction. Géophysique. Cycle Hercynien, 729 pp., ed. BRGM-ITGE, Orléans, France.

Bruker (2007). X-Ray Diffraction Software. Diffrac Plus Evaluation Software.

Cecinato, F., Zervos, A., Veveakis, E. (2011). A thermo-mechanical model for the catastrophic collapse of large landslides, Int. J. Numer. Anal. Meth. Geomech., 35, 1507-1535.

Cecinato, F., Zervos, A. (2012). Influence of thermomechanics in the catastrophic collapse of planar landslides. Can. Geotech. J., 49, No. 2, 207-225.

Chang, K., A. Taboada, M. Lin, and R. Chen (2005a), Analysis of landsliding by earthquake shaking using a block-on-slope thermo-mechanical model: Example of jiufengershan landslide, central taiwan, Engin. Geol., 80, 151–163.

Chatterjee, N.D. (1973). Low-temperature compatibility relations of the assemblage quartz-paragonite and the thermodynamic status of the phase rectorite. Contributions to Mineralogy and Petrology, 42, pp. 259–271.

Chester, F. M. and Logan, J. M. (1987). Composite planar fabric of gouge from the Punchbowl Fault, California. Journal of Structural Geology, Vol. 9, No. 5/6, pp. 621-634.

Clariana, P. (2004). El sinclinori de Tor-Casamanya. Estudi estratigràfic i estructural. pp. 1-3.

Coelho, A. A. (2000). Whole-profile structure solution from powder diffraction data using simulated annealing. Journal of Applied Crystallography, 33(3 Part 2), 899–908, doi:10.1107/S002188980000248X.

Collettini, C., Niemeijer, A., Viti, C., Marone, C. (2009). Fault zone fabric and fault weakness, Nature, 462(7275), 907–910, doi:10.1038/nature08585.

Corominas, J. (1990). Influencia del glaciarismo cuaternario en la estabilidad de laderas del valle del Valira d'Orient (Andorra). I Reunión Nacional de Geomorfología. Teruel. pp. 521-532.

Corominas, J., Alonso, E. (1984). Inestabilidad de laderas en el Pirineo catalán: tipología y causas. Jornadas Inest. Laderas en el Pirineo. C1-C53. Barcelona

Corominas, J., Moya, J., Lloret, A., Gili, J. A., Angeli, M. G., Pasuto, A., Silvano, S. (2000). Measurement of landslide displacements using a wire extensometer. Engineering Geology, 55, 149-166.

Corominas, J., Iglésias, R., Aguasca, A., Mallorquí, J.J., Fàbregas, X., Planas, X., Gili, J.A. (2013). Consideraciones sobre el deslizamiento del Forn de Canillo (Principa d'Andorra) a partir de datos de interferometría radar y nuevas observacions sobre el terreno. VIII Simposio Nacional sobre Taludes y Laderas Inestables. Palma de Mallorca, Junio 2013.

Corominas, J., Iglesias, R., Aguasca, A., Mallorquí, J.J., Fàbregas, X., Planas, X., Gili, J.A. (2015). Comparing Satellite Based and Ground Based Radar Interferometry and Field Observations at the Canillo Landslide (Pyrenees). Engineering Geology for Society and Territory, Vol. 2.

DeBlasio, F., and A. Elverhoi (2008), A model for frictional melt production beneath large rock avalanches., J. Geophys. Res., 113, F02,014, doi:10.1029/2007JF000867.

Euroconsult, S.A. (2002). Estudi de la zonificació del terreny segons la seva problemàtica geològica. Gran moviment del Forn. Parròquia de Canillo. Govern d'Andorra.

Euroconsult. (2007). Campañas testificación de los sondeos del deslizamiento del Forn de Canillo (Andorra).

Euroconsult. (2008 to 2018). Auscultacions inclinomètriques, piezomètriques y extensomètriques al moviment de massa del Forn.

Ferri, F., Di Toro, G., Hirose, T., Han, R., Noda, H., Shimamoto, T., Quaresimin, M., De Rossi, N., (2011). Low- to high-velocity frictional properties of the clay-rich gouges from the slipping zone of the 1963 Vaiont slide, northern Italy. Journal of Geophysical Research, Vol. 116, B09208.

Frey, M. (1969). A mixed-layer paragonite/phengite of low-grade metamorphic origin. Contributions to Mineralogy and Petrology, 14, pp. 63–65.

Frey, M. (1978). Progressive low-grade metamorphism of a black shale formation, Central Swiss Alps, with special reference to pyrophyllite and margarite bearing assemblages. Journal of Petrology, 19, pp. 95–135.

Frey, M. (1987a). Very low-grade metamorphism of clastic sedimentary rocks. In: Frey, M. (Ed.): Low Temperature Metamorphism. Blackie, Glasgow, pp. 9–58.

Gioretti, G., Monecke, T., Kleeberg, R., Herzig, P. M. (2003). Intermediate sodium-potassium mica in hydrothermally altered rocks of the Waterloo deposit, Australia: a combined SEM-EMP-XRD-TEM study. Contrib Mineral Petrol, 146: 159-173.

Goren, L., Aharonov, E. (2007). Long runout landslides: The role of frictional heating and hydraulic diffusivity. Geophys. Res. Lett., 34, No. L07301, 1-7.

Goren, L., Aharonov, E., Anders, M. H. (2010). The long runout of the Heart Mountain landslide: Heating, pressurization, and carbonate decomposition. J. Geophys. Res.: Solid Earth 115, No. 10B, 1-15.

Gupta, S., Fareeduddin (2013). Occurrence of paragonite in the hydrothermal wall rock alteration zone of G.R.Halli gold deposit, Chitradurga Schist Belt, Western Dharwar Craton, southern India. Journal of the Geological Society of India, 82, pp. 461-473.

Hartevelt, J.J.A. (1970). Geology of the Upper Segre and Valira valleys, Central Pyrenees, Andorra/Spain. Leidse.Geol.Med. 45:167-236.

Jaqués, I. (2014). Revisión del deslizamiento de Canillo, Andorra. Bachelor’s Thesis. Departamento de Ingeniería del Terreno, Cartogràfica y Geofísica. Barcelona: Universitat Politècnica de Catalunya.

Jaqués, I. (2015). Análisis geomecánico del deslizamiento del Forn de Canillo (Andorra) mediante el programa de elementos finitos Plaxis. Master’s Thesis. Departamento de Ingeniería del Terreno, Cartogràfica y Geofísica. Barcelona: Universitat Politècnica de Catalunya.

Jiang, W-T., D.R. Peacor (1993). Formation and modification of metastable intermediate sodium potassium mica, paragonite and muscovite in hydrothermally altered metabasites from northern Wales. American Mineralogist, 78, pp. 782–793.

Kilburn, R.J.C. and Petley, D.M., (2003). Forecasting giant, catastrophic slope collapse: lessons from Vajont, Northern Italy, Geomorphology, 54, 21-32.

Labaume, P., Maltman, A. J., Bolton, A., Tessier, D., Ogawa, Y., Takizawa, S. (1997). Scaly fabrics in sheared clays from the décollement zone of the Barbados accretionary prism. Proceedings of the Ocean Drilling Program, Scientific Results, Vol. 156.

Lachenbruch, A. H. (1980). Frictional heating, fluid pressure and the resistance to fault motion. J. Geophys. Res. 85, 6097-6112.

Li, G., D.R. Peacor, R.J. Merriman, B. Roberts (1994). The diagenetic to low-grade metamorphic evolution of matrix white micas in the system muscovite-paragonite in a mudrock from Central Wales, United Kingdom. Clays and Clay Minerals, 42, pp. 369–381.

Livi, K.J.T., D.R. Veblen, J.M. Ferry (1990). Segregation of K- and Na-rich micas in low-grade metamorphosed shale from the Liassic black shale, Switzerland. IGCP Project No. 294: Very low-grade metamorphism. Conference on Phyllosilicates as Indicators of Very Low Grade Metamorphism and Diagenesis. Programme and Abstracts. Manchester, July 1990.

Livi, K.J.T., D.R. Veblen, J.M. Ferry, M. Frey (1997). Evolution of 2:1 layered silicates in low-grade metamorphosed Liassic shales of Central Switzerland. Journal of Metamorphic Geology, 15, pp. 323–344.

Maltman, A. J. (1987). Shear zones in argillaceous sediments – an experimental study. Geological Society, London, Special Publications, 29 (1):77.

Mase, C.W. and Smith L. (1984). Pore-fluid pressures and frictional heating on a fault surface, PAGEOPH, Vol. 122 (2-4), 583-607, doi: 10.1007/BF00874618.

Mineral Data Publishing (2001). Paragonite. Version 1.2.

Regenauer-Lieb, K., D. Yuen, and F. Fusseis (2009), Landslides, ice quakes, earthquakes: A thermodynamic approach to surface instabilities, Pure. Appl. Geophys, 166(10-11), 1885–1908.

Rietveld, H. M. (1969), A profile refinement method for nuclear and magnetic structures, Journal of Applied Crystallography, 2(2), 65–71, doi:10.1107/S0021889869006558.

Rice, J., (2006). Heating and weakening of faults during earthquake slip, J. Geophys. Res., 111, B05311, doi: 10.1029/200JB004006.

Planas, X., Corominas, J., Vilaplana, J.M., Altimir, J., Torrebadella, J., Amigó, J. (2011). Noves aportacions del gran moviment del Forn de Canillo, Principat d'Andorra. Resúmenes XIII Reunión Nacional de Cuaternario, Andorra la Vella. 5pp.

Santacana, N. (1994). Estudi dels grans esllavissaments d'Andorra: Els casos del Forn i del vessant d'Encampadana. En Tesis de Licenciatura. Departamento de Geología, Dinàmica, Geofísica i Paleontología de la Facultad de Geología de la UB. pp. 1-169.

Segui, C., H. Rattez, and E. Veveakis (2019), On the stability of deep-seated landslides. the cases of vaiont (italy) and shuping (three gorges dam, china), Journal of Geophysical Research, under re- view(https://doi.org/10.31223/osf.io/ukbr5).

Soutadé, G. (1988). Le Glissement d'el Forn. II Simposio sobre Taludes y Laderas Inestables. Andorra la Vella, 9-11 de Marzo de 1988. pp.1-15.

Tchalenko, J. S. (1968). The evolution of kink-bands and the development of compression textures in sheared clays. Tectonophysics, 6 (2), 159-174.

TJP (2015). Avizo Guide. Avizo training materials, BRC-Imaging Facility.

Torrebadella J, Villaró I, Altimir J, Amigó J, Vilaplana JM, Corominas J, Planas X (2009) El deslizamiento del Forn de Canillo en Andorra. Un ejemplo de gestión del riesgo geológico en zonas habitadas en grandes deslizamientos (in Spanish). In: Proceed. VII Simposio Nacional sobre Taludes y Laderas Inestables, held in Barcelona, Oct 2009. Ed. CIMNE, UPC, pp 403–414.

Vardoulakis, I. (2000), Catastrophic landslides due to frictional heating of the failure plane., Mech. Cohesive Frict. Mater., 5, 443–467.

Vardoulakis, I., (2002a). Dynamic thermo-poro-mechanical Analysis of Catastrophic Landslides, Géotechnique, 52, 157-171.

Veveakis, E., Vardoulakis, I., and Di Toro, T., (2007). Thermoporomechanics of creeping landslides: The 1963 Vaiont slide, northern Italy. J. Geophys. Res., 112, F03026, doi: 10.1029/2006JF000702.

Voight, B., and Faust, C., (1982). Frictional heat and strength loss in some rapid landslides, Géotechnique, 32, 43-54.

Zwart, H. J. (1979). The Geology of the Central Pyrenees.Leidse geologische mededelignen, ISSN 0075-8639.

Figure 8. Combination of all the results performed in the study and its evolution, organized following the stratigraphic column (shallowest at the top to deepest at the bottom). For each sample, the figure shows: thin section image, a representative SEM-EDS image, mineralogy and its percentage, porosity, plasticity index, and displacement from April to June of 2017 [Euroconsult, 2017] of the landslide at each corresponding depth.